Miniature Digital Load cell weight sensor 1kg.

Description

Four leads, easy to use,5-10V drive voltage applied, the force is directly output as a voltage change signal.

Dimensions: 12.7mm * 12.7mm * 80mm

Sensor through the screw hole at one end fixed and the other end to keep floating state, applying gravity following the direction indicated by label.

Particular attention must not directly push the white plastic cover part, so as not to damage the sensor.

Application: Kitchen scale

Module: YZC-133

Capacity: 1、5、10、20

Rated Output: 1.0±0.15mV/V

Nolinearity: 0.0005

Repeatability: 0.0003

Hysteresis: 0.0003

Creep(5Min): 0.001

Temperature Effect On Sensitivity: 0.003%RO/°C

Temperature Effect On Zero: 0.02%RO/°C

Zero Balance: ±0.1%RO

Input Resistance: 1066±20Ω

Output Resistance: 1000±20Ω

Insulation Resistance: 2000MΩ(50V)

Recommended Excitation Voltage: 5V

Compensated Temperature Voltage: -10~+50°C

Operating Temperature Range: -20~+65°C

Safe Overload: 120%RO

Ultimate Overload: 150%RO

Load Cell Material: Aluminium

Connecting Cable: ?0.8X180mm

Connection:

Red to E+

Black to E-

Green to A+

White to A-