## 1️⃣ Components Overview

| Component | Role |
| --- | --- |
| Scheduler (sched.c) | Core timing engine; triggers events at precise times (step pulses, timers). |
| Stepper module (stepper.c) | Queues steps for X/Y/Z/E axes; handles STEP/DIR pins; uses scheduler to precisely time pulses. |
| TMC UART driver (tmc.c / tmc_uart.c) | Configures TMC drivers, reads registers (e.g., stallGuard), adjusts current dynamically; communicates over hardware UART using async queues. |
| Main firmware loop | Processes G-code commands, motion planning, status reporting. |

## 2️⃣ Flow of Events

- Firmware Startup

  - Scheduler initializes.

  - Stepper module initializes axes.

  - TMC UART drivers initialize: writes GCONF, IHOLD_IRUN, CHOPCONF.

- Motion Planning

  - Main loop receives G-code or motion commands.

  - Planner calculates step pulses and timing.

- Queue Steps

  - Stepper module enqueues steps per axis.

  - Scheduler sets timers for each step event.

- Stepper Execution

  - Scheduler fires step timers → stepper module toggles STEP pins.

  - STEP/DIR pulses move motors physically.

- TMC UART Interaction (Async)

  - UART driver maintains write/read queue.

  - ISR triggers whenever UART FIFO is ready.

  - Writes: configure driver registers (current, microsteps, StealthChop).

  - Reads: poll SG_RESULT or DRV_STATUS for sensorless homing or fault detection.

  - Updates internal driver state structures asynchronously.

- Feedback Loop

  - StallGuard or driver errors trigger software actions:

    - Stop motion

    - Adjust motor current

    - Report errors to host

- Continuous Scheduler + UART

  - Scheduler handles step timing independently of UART queue.

  - UART ISR runs asynchronously, never blocking motion.

## 3️⃣ ASCII Diagram

┌──────────────────────────┐

│       Main Loop          │

│  (G-code + Planner)     │

└─────────┬────────────────┘

│

▼

┌──────────────────────────┐

│       Stepper Module     │

│  Queues steps per axis   │

└─────────┬────────────────┘

│

▼

┌──────────────────────────┐

│       Scheduler          │

│ Fires step timers        │

│ Executes step pulses     │

└─────────┬────────────────┘

│

┌────────────────┴───────────────┐

▼                                ▼

┌───────────────────┐            ┌───────────────────┐

│ STEP/DIR Pins     │            │ TMC UART Driver    │

│ Motor pulses      │            │ Async write/read   │

│ Moves motors      │            │ Queued via ISR     │

└───────────────────┘            │ Writes: GCONF, IHOLD_IRUN, CHOPCONF

│ Reads: SG_RESULT, DRV_STATUS

└─────────────┬───────┘

▼

┌────────────────┐

│ Driver State    │

│ Cached registers│

│ StallGuard, etc.│

└────────────────┘

## 4️⃣ Key Insights

- Scheduler and UART driver are independent but work in parallel.

- Stepper pulses are precise; UART communication is async.

- TMC feedback (stallGuard, error flags) is polled asynchronously, allowing dynamic adjustments.

- This design allows high-speed, low-latency motion without blocking on serial commands.

Perfect! Let’s create a timing diagram showing how Klipper handles step pulses and TMC UART commands asynchronously. This will make it clear how motion and UART interleave in real time.

## 1️⃣ Timeline Key

- Time → left to right

- STEP[X/Y/Z/E] — step pulses for each axis

- Scheduler Timer — triggers step pulses

- UART Queue / ISR — async register writes or reads

- TMC Driver — processes UART packets and updates internal state

## 2️⃣ ASCII Timing Diagram

Time ─────────────────────────────────────────────────────────────►

Scheduler Timer: ─┐      ┌      ┌      ┌      ┌      ┌      ┌

▼      ▼      ▼      ▼      ▼      ▼      ▼

STEP X:           ▓      ▓      ▓      ▓      ▓      ▓      ▓

STEP Y:           ▒         ▒         ▒         ▒         ▒

STEP Z:               ░         ░         ░         ░

STEP E0:                  ▓         ▓         ▓         ▓

UART Queue: ──────> [WRITE GCONF X]  [WRITE IHOLD_IRUN X]

[WRITE GCONF Y]  [READ SG_RESULT Z]

[WRITE CHOPCONF E0]

UART ISR:        ↑         ↑          ↑           ↑

│         │          │           │

└────────> Sends packets asynchronously

Updates driver state when responses arrive

TMC Driver:     ───┐            ┌─────────────┐

│ Executes write / read

│ Updates cached registers

└─────────────┘

## 3️⃣ How to Read the Diagram

- Scheduler Timer fires repeatedly at microsecond/millisecond intervals to produce precise step pulses.

- STEP[X/Y/Z/E] signals move the motors; these pulses are independent of UART timing.

- UART Queue holds pending writes and reads for the TMC drivers.

- UART ISR sends one packet at a time whenever UART is free.

- TMC Driver executes the command, updating internal state (current, microsteps, StealthChop, or SG_RESULT).

Key points:

- Step pulses are high-priority and precise.

- UART communication is low-priority and asynchronous, handled by the ISR.

- Feedback from drivers (stallGuard, error flags) is read asynchronously and can dynamically affect motion if needed.

- This decoupling allows high-speed motion with live driver monitoring, exactly like Klipper.

Perfect! Let’s make a full multi-axis Klipper-style interaction map showing:

- Multiple stepper axes (X/Y/Z/E)

- Scheduler timing

- Async UART writes/reads for multiple TMC drivers

- Feedback loops from SG_RESULT / DRV_STATUS

- How motion and UART interleave

This is essentially a complete roadmap of motion + TMC UART in real-time.

## Full Multi-Axis Interaction Map (ASCII)

Time ─────────────────────────────────────────────────────────────────────────►

Scheduler Timer: ─┐      ┌      ┌      ┌      ┌      ┌      ┌      ┌

▼      ▼      ▼      ▼      ▼      ▼      ▼      ▼

STEP X:           ▓      ▓      ▓      ▓      ▓      ▓      ▓      ▓

STEP Y:           ▒         ▒         ▒         ▒         ▒

STEP Z:               ░         ░         ░         ░

STEP E0:                  ▓         ▓         ▓         ▓

STEP E1:                       ▒         ▒         ▒

UART Queue: ─────────────> [WRITE GCONF X]

[WRITE IHOLD_IRUN X]

[READ SG_RESULT X]

[WRITE GCONF Y]

[WRITE IHOLD_IRUN Y]

[READ SG_RESULT Y]

[WRITE GCONF Z]

[READ SG_RESULT Z]

[WRITE CHOPCONF E0]

[WRITE CHOPCONF E1]

UART ISR:      ↑         ↑           ↑           ↑           ↑

│         │           │           │           │

└────────> Sends packets asynchronously to respective TMCs

Reads responses and updates driver state

TMC Drivers: ──────┐         ┌───────────┐         ┌───────────┐

│ Executes writes/reads

│ Updates cached registers

│ SG_RESULT / DRV_STATUS polled asynchronously

└───────────┘

Driver State Updates:

- SG_RESULT_X → may trigger sensorless homing stop

- SG_RESULT_Y → dynamic current adjustment

- DRV_STATUS_Z → error detection

- StealthChop enabled/disabled dynamically per axis

Motion & UART Decoupling:

- Step pulses (STEP pins) continue independently of UART queue

- Scheduler guarantees timing precision

- UART ISR handles driver communication asynchronously

### Key Insights

- Multiple axes are handled independently, each with its own step pulses and TMC driver.

- Async UART queue allows multiple writes/reads to different drivers without blocking motion.

- Driver feedback (stallGuard, errors) can dynamically affect motion in real-time.

- Scheduler ensures precise step timing, completely decoupled from UART timing.

- Klipper’s efficiency comes from this separation: high-speed motion + live driver communication.

Perfect! Let’s create a visual timeline (Gantt-style) showing step pulses and TMC UART packets interleaving across multiple axes. This will make it crystal clear how motion and UART operate in parallel in Klipper.

## Multi-Axis Motion + TMC UART Timeline (ASCII/Gantt Style)

Time ─────────────────────────────────────────────────────────►

Scheduler Timer: ─┐      ┌      ┌      ┌      ┌      ┌      ┌

▼      ▼      ▼      ▼      ▼      ▼      ▼

STEP X:           ▓      ▓      ▓      ▓      ▓      ▓      ▓

STEP Y:           ▒         ▒         ▒         ▒         ▒

STEP Z:               ░         ░         ░         ░

STEP E0:                  ▓         ▓         ▓         ▓

STEP E1:                       ▒         ▒         ▒

UART Queue: ──────────────────────► [WRITE GCONF X] ──┐

[WRITE IHOLD_IRUN X] ──┐

[READ SG_RESULT X]      │

[WRITE GCONF Y]         │

[WRITE IHOLD_IRUN Y]    │

[READ SG_RESULT Y]      │

[WRITE GCONF Z]         │

[READ SG_RESULT Z]      │

[WRITE CHOPCONF E0]     │

[WRITE CHOPCONF E1]     │

UART ISR:        ↑          ↑           ↑           ↑           ↑

│          │           │           │           │

└───────── Sends packets asynchronously to TMC drivers

(one at a time, FIFO order)

TMC Drivers: ──────┐            ┌───────────┐            ┌───────────┐

│ Executes write / read

│ Updates cached registers

│ SG_RESULT / DRV_STATUS polled asynchronously

└───────────┘

Driver Feedback Effects:

- StallGuard_X → may stop X motion if sensorless homing detected

- StallGuard_Y → dynamically adjust current or detect stall

- DRV_STATUS_Z → triggers error handling

- StealthChop toggled per axis dynamically

Main Insights:

1. **STEP pulses are high-priority**, triggered precisely by the scheduler.

2. **UART ISR is low-priority**, asynchronous, does not block motion.

3. **Multiple axes share the UART queue**, handled sequentially by ISR.

4. **Driver feedback is processed asynchronously**, can influence motion safely.

### How to Read This Timeline

- Horizontal bars (▓ ▒ ░) = step pulses for each axis

- UART queue entries = queued TMC writes/reads, executed asynchronously

- UART ISR arrows = actual packet transmission

- TMC Drivers = receive commands, update registers, and provide feedback

This is essentially Klipper’s architecture in action:

- Motion planner → Scheduler → Stepper pulses

- UART commands → async queue → ISR → TMC driver

- Feedback loop integrates seamlessly without blocking step pulses