### Specification

| Item Name | Incremental rotary encoder |
| --- | --- |
| Model | E6B2-CWZ6C |
| Material | lead alloy (shell), aluminum (body) |
| Input voltage | DC5 ~ 26V(open-collector output type) |
| Output mode | voltage output, collector output, complementary output, push pull output, long line drive output, differential output, fork output |
| Output phase | A, B, Z phase |
| Outer diameter | 38 mm |
| Thrust shaft diameter | 6mm |
| Standard length of wiring wire | 2m |
| Resolution (pulse/rotation) | 100, 200, 300, 360, 400, 500, 600, 1000, 1024, 1200, 2000, 2500, 3600, 5000 (optional) |

Wiring method:

Brown line: DC5-26V (positive pole)

Blue line: 0V (negative pole)

Black line: A(phase)

White line: B(phase)

Orange line: Z(phase)

Shielding network cable: N.C (ground)

Phase Difference Between Outputs: 90°±45° between A and B (1/4 T ± 1/8 T)

Output Configuration: NPN open-collector output

Output Capacity: applied voltage: 30 VDC max.; synchronous current: 35 mA max.; residual voltage: 0.4 V max.(at synchronous current of 35 mA)

Maximum Response Frequency(*3): 100kHz

Rise and Fall Times of Output: 1 μ s max. (Control output voltage: 5 V, Load resistance: 1 kΩ, Cable length: 2 m max.)

Starting Moment of Force: 0.9 mN. m max.

Moment of Inertia: 1x10-6 kg. m ² max.; (3x10-7 kg.m ² max. at 600 P/R max.)

Radial Shaft Loading: 30N

Thrust Shaft Loading: 20N

Maximum Permissible Rotating Speed: 6,000r/min

Protection Circuits: load short circuit protection, power supply reverse wiring protection

Ambient temperature range: operating: -10 to 70 ℃; storage: -25 to 85℃ (with no icing)

Operating/Storage Ambient Humidity Range: 35% to 85% (with no condensation)

Insulation Resistance: 20 MΩ min. (at 500 VDC) between current-carrying parts and case

Dielectric Strength: 500 VAC, 50/60 Hz for 1 min between current-carrying parts and case

Vibration Resistance(Destruction): 10 to 500 Hz, 2mm or 150 m/s ² double amplitude for 11 min 3 times each in X, Y, and Z directions

Shock Resistance(Destruction): 1,000m/s ² 3 times each in X, Y, and Z directions